Engineered Recycled Rigid PVC

Exterior Wall Submittal

07 46 33 ChamClad Laminated Film PVC Siding Panels

Exterior 6” Classic Wall Panel and Shadowline Series 6” Panel with 6” Reveal,

6” Panel with 3” Reveal, 12” Panel with 2” Reveal

Product Specification Information:

07 46 33 ChamClad Laminated Film PVC Siding Panels

07 42 93 ChamClad Laminated Film PVC Soffit Panels

09 54 55 ChamClad Laminated Film PVC Interior Ceiling Panels

09 78 25 ChamClad Laminated Film PVC Interior Wall Panel

March 22, 2024

CAN

info@chamclad.com

Chamclad.com

780-456-4430

Table of Contents:

Install Manual 2

Profile Drawings & Wall Assembly 63

Specification Document 65

- Technical Spec Sheets 71

- Colors 73

Warranty 74

Wall Panel

March 11, 2024

USA

CAN

# Installation Manual

ChamClad®

CCMC 14323-R

ICCES ESR 4560

Revised: March 22nd, 2024

Supersedes January 2024

2020

Table of Contents

Table of Contents 3

Before you Begin 4

First Steps for Successful Installation 5-6

Best Practices, Recommended Fasteners and General Installation 7-10

Expansion/Contraction & Gap Guide 11-13

ChamClad® Exterior Architectural Products 14

Exterior Horizontal Wall Panel Installation 23

Exterior Vertical Wall Panel Installation 25

Exterior Soffit and Fascia Installation 27

ChamClad® Interior Architectural Products 31

Horizontal Interior Installation 34

Interior Vertical Installation 35

Interior Ceiling Installation 36

Column and Beam Wrap Installation 37

Assembly Drawings 41-53

Care and Maintenance 54

Installation Tips - At a glance 55

Glossary 59

QR CODE- For Install Videos on Both Soffit and Wall.

BEFORE YOU BEGIN

- Read the install instructions completely. These installation instructions illustrate the “Best Practices” to install ChamClad® cladding and trim.

ChamClad® does not accept any liability or responsibility under their written warranty for product failure caused by improper installation.

- To ensure you are up to date with the most recent install instructions, please visit our website https://ChamClad.com and view manual in conjunction with the technical bulletin advisory.

- Our research and development team are committed to improving product durability and performance. As a result, specifications may change from time to time. Please check our website for updated technical information and profile drawings.

- No one can anticipate all questions or conditions which may arise during installation. This Installation Manual is focused on methods, tools, and techniques used for a typical install. Should you encounter something not covered in this manual, please contact ChamClad® customer service by email. insidesales@chamclad.com

- Occasionally you may come across minor irregularities that happen naturally during the manufacturing process. This occurrence will not affect the aesthetics and performance of the product.

First Steps for a Successful Installation

- Inspect all material for any breakage, defects, damage, and colour before installing. Do not install product that is questionable. Contact ChamClad® customer service or your dealer for assistance. Installing defective material may void your product Warranty.

- Before installing ChamClad® cladding it is very important that installers read and understand the installation recommendations and topics covered in this manual.

ChamClad® Transportation, Proper Handling, and Storage

When transporting ChamClad® products, keep bags flat. The boards must be supported along the full length. Whenever possible avoid using metal banding. Metal banding will cut into the boards if pulled too tight. Materials are required to be inspected for any accidental damage within 7 days of receipt, recorded on the Bill of Lading, and forwarded to info@ChamClad.com. ChamClad® warranty may be voided if damaged product is installed on the wall without prior inspection by the installer.

Proper Handling and Storage

Always carry ChamClad® cladding by holding it by the narrow edge and pick boards up in the center. ChamClad® pallets should not be stacked more than 2 pallets high and always stored on a flat surface.

Storage of ChamClad®

- Do not place or store on asphalt or in excessive heat.

- Store panels on a flat surface that supports the entire length of the panels.

- Product should be kept dry and out of direct sunlight for long periods of time.

- ChamClad bags are designed with ventilation and a white exterior to effectively reflect heat and promote airflow. This feature is particularly beneficial when ChamClad products are exposed to direct sunlight for extended periods, such as during storage rather than active use. To ensure optimal performance, it is recommended to cover the product with a tarp while leaving the ends open to allow for adequate airflow.

- For warranty coverage, please ensure all installation practices are followed according to the installation manual.

Cutting and Tools

- Always ensure you wear proper, appropriate and standard PPE’s when performing cutting and installation.

- If using a circular saw, cut cladding face down on a clean surface.

- If using a cut-off saw, cut face up

- Carbide-tipped blades with 100+ teeth are recommended. (Aluminum blades are optimal)

- Do not score or snap ChamClad® exterior cladding

- Do not cut ChamClad® with coarse saw blade to avoid chipping or damaging the panel

BEST PRACTICES GUIDE

INTRODUCTION

These installation instructions illustrate the “Best Practices” to install ChamClad® cladding, and trim.  ChamClad® does not accept any liability or responsibility under their written warranty for product failure caused by improper installation.

It is the responsibility of the installer to observe the requirements of the local Building Code as they relate to installation in any given area. Most Building Codes are derived from National Building Codes. In Canada, the primary model is the National Building Code (NBC), which is published by the National Research Council of Canada (www.nrc-cnrc.gc.ca). In the United States, the primary model is based on the International Building Code (IBC), which produces the International Residential Building Code (IRC).

It is the responsibility of the builder or contractor, to understand the local code and how it pertains to the location where ChamClad® is being installed.

GENERAL INSTALLATION REQUIREMENTS

Fasteners

See page 9 for alternative screw examples.

Fastener Size is critical to a successful ChamClad® installation; we recommend the following:

Screws: It is strongly recommended to use ChamClad® universal screws which are available from your local distributor or directly from ChamClad®.

The fasteners must penetrate the wall to a minimum depth of 1 ¼” and allow for the expansion and contraction of the ChamClad® products. If using metal furring strips, use of self-tapping screw is required.

If ChamClad® fasteners unavailable, use low profile, corrosion resistant or stainless-steel screws long enough to fasten through the pre-punched flange. Do not use electroplated fasteners. It is important that the fastener head can fit inside the nailing slot. Fastener heads that protrude outside of the nailing slot will hinder ChamClad® tongue and groove locking mechanism.

ChamClad® Universal Screw

#8 quad-drive 1 ½”

Self-tapping screw with 1/8” threadless shaft

Examples of alternate screws:

Exterior Wall

17 Self tapping

Round Washer Lo-root

#8 1 ½”

Soffit & Trims

Modified Truss Head

#8 1 ¼”

Note: DO NOT USE a stapler, or nail gun for exterior cladding and soffit panel installation. GLUE IS NOT to be used to fasten strapping or ChamClad® exterior wall boards.

Random ChamClad® Panel Pattern

ChamClad® wall panels are laminated with a repeating pattern (48”). ChamClad® makes every effort to ensure the patterns are mixed in each bag to minimize repeat patterns.  It is suggested to randomly check panels at the job site for repeat patterns and adjust as required.  To avoid excessive repetition when cutting boards 4’ or less, use 2 boards with different patterns.

Classic Panel (1 Panel, 2 Looks)

ChamClad® offers a simple tongue and groove system with the product stacking from the bottom up. For a U-Channel profile, insert to the first stop. For a V-Groove profile, take a non-marking rubber mallet and tap to the next stop. Does not apply for Shadow Line Panel.

Air Barrier/ Rainscreen

ChamClad® exterior wall panels can be installed over many weathers resistant sheet membranes. It is recommended as per CCMC/ ICCES that 6” exterior claddings are installed on an air barrier/ rainscreen. ChamClad® panels may be installed over wood furring strips (min. 7/16”), galvanized steel z-girt (min. 20 ga.) or a continuous 3-dimensional membrane such as home slicker, as per Building Code requirements. ChamClad® exterior claddings must be fastened every 16-24” O.C. to strapping.

Wall Flashings and Penetrations

Windows, doors, and utility penetrations must be flashed and trimmed properly. It is the responsibility of the installer/designer/builder to ensure it is done to local codes and best industry practice. If sealants are required, ChamClad® recommends ASTM 920 Elastomeric Joint Sealants (e.g., Tremco Spectrum 2).

2pc Trim - Vertical Installation

When installing 2pc trims vertically (2pc Outside Corner, 2pc J Trim, 2pc H-Trim), in order to prevent downward migration, it is recommended to fasten the base trim at the highest point of the nail slot with all other fasteners located in the centre of the remaining slots (every 8-12”).

Before applying the top cap, it is recommended to add a small dab of sealant (1”) between the teeth at the top and bottom of the base trim. This is especially important around doors and windows where you are not completing a full perimeter wrap.

Compatibility

ChamClad® products are not compatible with asphalt or products containing asphalt. Do not store bags on asphalt (i.e., driveways or roads) as panels and asphalt should not be in contact with each other. Asphalt, if brought into contact with ChamClad® will leach into the panels and possibly weaken or discolour the product. If ChamClad® products come into contact with asphalt, clean the ChamClad® product immediately with a non-solvent cleaning product.

Temperature Changes & Expansion/Contraction

It is important to remember that ChamClad® materials will expand and contract with regular temperature changes. ChamClad® trims allow for any variance in panel lengths due to expansion/contraction.

When placing fasteners in the center of the designated nail slots, always leave a 1/32” gap between the fastener head and nail flange to allow ChamClad® panels expand/contract as needed.

The below guide provides expansion/contraction examples based on different panel lengths and temperature fluctuations.

Expansion Guide (by temperature shift)

| Temperature Change in °Celsius and Fahrenheit |  | Length of Panel (feet) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 12' | 20' | 30' | 35' |
| 10 | 50 | 0.10 | 0.17 | 0.25 | 0.29 |
| 20 | 68 | 0.20 | 0.34 | 0.50 | 0.59 |
| 30 | 86 | 0.30 | 0.50 | 0.75 | 0.88 |
| 40 | 104 | 0.40 | 0.67 | 1.01 | 1.17 |
| 50 | 122 | 0.50 | 0.84 | 1.26 | 1.47 |
| 60 | (Optimal Gap) 140 | 0.60 | 1.01 | 1.51 | 1.76 |
| 70 | 158 | 0.70 | 1.17 | 1.76 | 2.05 |
| 80 | 176 | 0.80 | 1.34 | 2.01 | 2.35 |
| ChamClad® 's Coefficient of Linear Expansion (CLE): 0.0000698 |  | Total Expansion/Contraction Per Panel (inches) |  |  |  |

Temperature Shift Example using ChamClad® products.

The change in length of any material because of expansion and contraction is dependent on 3 factors:

- The Coefficient of Linear Expansion (CLE) of the material.

For ChamClad® Panels, the CLE is 0.0000698 in/in/°C.

- The original length of the material at time of cut

- The net change in temperature

Change in length (inches) = (Original Length in inches) x (Temperature Change in °C) x (CLE in in/in/C units)

If there is a 12’ (144”) ChamClad® length that goes from 20°C (68°F) to 0°C (32°F), it will contract, theoretically, by this amount:

144 x 20 x 0.0000698 = 0.20” or 7/32”

For example, if installed in 20°C (68°F), the final length of a 144” long ChamClad® panel at 0°C (32°F) would become 143.80.”

When temperatures return to 20°C (68°F), it is expected that the PVC material will expand and return to 144” in length.

General Rules

Based on a temperature change of 40°C (104°F), for example: 0°C (32°F) in cooler months and reaching 40°C (104°F) in warmer months:

For panel lengths up to 20' leave a 1/2" gap per side; 20' or longer leave a 3/4” gap per side and use larger faced trims (3.5” 2pc Finish Trim, 3.5” 2pc Outside Corner, larger faced H-Trim).

It is imperative that ChamClad® panels are cut to fit the coverage of trim pieces.

The temperature the day of installation factors into the expansion and contraction gap you leave. If installing on a cooler day allow for more expansion and a larger gap, if installing on a warmer day allow for less expansion and a smaller gap. The below guide provides recommended spacing based on temperature and Panel Length.

Recommend Gap Allowance Temperature Guide

Guide Lines

- Take into consideration the temperature on date of installation and season you are installing in.

- Installation in weather colder than -10°C (14°F) is not recommended due to potential chipping or cracking of the panels. Should that occur warranty may be voided.

- Generally speaking, in the colder months, leave a gap of ½” for panels up to 20’ and ¾” for panels over 20’. It is recommended to use the large faced trims to allow for appropriate expansion and contraction.

    - Example: When installing at -10°C (14°F) leave a gap of 5/8” for 20’ panels; ½” for 16’ panels and 3/8” for 12’ panels.

- In the warmer months, leave a gap of 3/8” for panels up to 20’ and ½” for panels over 20’.

    - Example: When installing at 20°C (68°F) leave a gap of 3/8” for 20’ panels; 5/16” for 16’ panels and 1/4” for 12’ panels.

It is important to acclimate panels and accessories for a minimum of 24 hours prior to installation.

CHAMCLAD

EXTERIOR

WALL, SOFFIT, HORIZONTAL, VERTICAL

CHAMCLAD® EXTERIOR ARCHITECTURAL WALL PANELS

6” Classic Wall Panel and Shadowline Series

*Panels may be used in exterior soffit (venting by others), interior wall and ceiling

Classic Panel is one panel that offers two looks!

U Channel & V Groove

6” Classic Wall Panel ½”

6” Shadowline Wall Panel ½”

with 6” reveal

6” Shadowline with 3” Reveal Wall Panel ½”

12” Shadowline with 2” Reveal Wall Panel ½”

CHAMCLAD® Exterior Wall Trims

*2pc 3.5” Outside Corner Trim

10’ & 20’

*2pc 2” Outside Corner Trim

10’ & 20’

*2pc 2” J Trim

12’ & 20’

*2pc 3.5” Finish Trim

12’ & 20’

| Starter Strip 12’, 16’, & 20’ Inside Corner Trim 10’, 12’, & 20’ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

2pc 2.75” Aluminum H-Trim

12’ & 20’

PVC Universal J Trim (1 pc)

12’ & 20’

*THESE TRIMS ARE AVAILABLE IN ALL ALUMINUM OR HYBRID COMBINATIONS

COLUMN AND BEAM WRAPS

1” x 4” Architectural Trim

Quick Click Column Cap and Base Trim

Available in 7”, 8”, 10”, and 12”

7” 2pc Column Wrap

Beam Bracket

2pc Beam 5.5”x 3.35”

Beam End Cap

Quick Click Column Cap and Base Trim

Available in 7”, 8”, 10”, and 12”

4pc Quick Click Column and Beam Wrap

Available in 8”, 10”, and 12”

New Summer 2024

Spray Bombs

Colour matched Spray Paint.

Recommended for use with the Column Wrap Trims or any larger surface areas.

ACCESSORIES

ChamClad® Universal Screw

#8 quad-drive 1 ½”

Self-tapping screw with 1/8” threadless shaft

Touch Up Pens

PREPARATION: The areas to be touched up must be dry and free from dust, dirt, and grease. Before each use, shake Pen for approximately 30 seconds. Rattle of mixing balls must clearly be heard.                                                                                                                                                             DE-AERATION: Due to exterior influences (heat) an overpressure may develop. To prevent blotching, the pen must be de-aerated. Put pen with tip pointing downwards onto a solid, non-soaking surface and press down – hold for 2 seconds, release and repeat procedure until tip is filled with touch- up (7-10 times may be necessary).

APPLICATION: Touch down pen tip at a flat angle (about 45 degrees) using light pressure. The valve opens and touch-up flows continuously. With the new designed tip, you can work precisely on hard-to- reach areas. Different stroke widths can be easily applied. Touch up rapidly with one stroke using light pressure. Important: Do not apply another coat of touch up until first coat has dried (approximately 3-5 minutes). If opacity decreases, shake pen again and de-aerate as described above. If touch-up flow decreases, simply pump again. Put cap on immediately after use – cap has to snap with a “click”. Over time, tip may become blocked with pigment sediments. If this occurs, replace tip with a new one, shake Pen well and repump.

*Do not let pens freeze

EXTERIOR HORIZONTAL WALL PANEL INSTALLATION

6” Classic and Shadowline Series

(See assembly drawings listing on page 39)

- Acclimate panels and accessories for a minimum of 24 hours prior to installation. With each course of ChamClad® panels, proper gaps must be left between the end of the wall panel and the track of the base trims for expansion and contraction.

    - Refer to the gap allowance temperature guide chart on page 10 before beginning.

- Use a level to produce a guideline around the entire building and mark with a chalk line.

Snap line where the starter strip will be installed.

- When installing air barrier/ rainscreen, use either furring/strapping (16-24” O.C.) or steel furring/strapping every (16-24” O.C.) vertically across the wall or a continuous 3-dimensional membrane (homeslicker)

- Install a ChamClad® Starter Strip fastening every 12” (1/32” gap from fastener head to nail flange to allow for horizontal expansion and contraction). Install panel 2” above grade or ½” above concrete (set Starter Strip accordingly) following local building codes.

- Install flashing and trim bases around windows, doors, utility box fixtures, and all other penetrations through the wall cladding. We recommend all ChamClad® accessories be fastened every 8-12” (8” in high heat or wind exposure areas). It is common industry practice to align all vertical and horizontal trims on the same wall to ensure a visually clean look.

- Fasten all panels 16-24” O.C. starting within 4” from both ends of panel. Ensure that screws are placed in the middle of the nail slots to allow for proper expansion and contraction. For panels over 24’ - Pin Panel at centre of nailing fin and not in nailing slots to equalize expansion and contraction.

- Begin installing your bottom courses of ChamClad® wall panels using ChamClad® Universal screws. When installing panel lengths 20’ and over, it is recommended to use the large, faced trims to allow for appropriate expansion and contraction. Check every three or four courses to ensure that ChamClad® wall panels are level and that adjacent panels are aligned horizontally.

- Except where designated in this manual, fasteners must be placed in and through the center of the nail slot. The fastener must be installed straight and not at any angles. The head of the screw or crown should allow for the thickness of a dime (roughly 1/32”) behind the face of the mounting hem to allow for expansion and contraction of the ChamClad® panels.

- If you need to apply a fastener in a section that does not have a manufacturers pre-drilled hole, you MUST pre-drill a nail slot to allow the panel to expand and contract without restriction. (Nail slot punch may be used).

- Application of all 2pc trim top caps is the last step to completing a ChamClad® panel installation. Ensure not to overdrive the trim tops to the point where panels can no longer expand and contract due to tightness.

- ChamClad® exterior panels may be used in beam, post, or column wrap applications. When installing horizontally begin with installation of the bases of the 2pc Outside Corner on all corners.  Continue with installing starter strip between corners at the base and 2pc J Trim between the corners at the top.  Install all courses of ChamClad® panels and complete with the installation of the 2pc Corner Cap and the 2pc J trim cap along the top. Refer to assembly drawing listing on page 39 .

Notes:

- It is recommended as per CCMC/ ICCES that 6” exterior cladding is installed on an air barrier/ rainscreen.

- DO NOT BUTT JOIN WALL BOARDS – you MUST use an H-Trim. Leave a proper gap inside the H-Trim to allow for expansion and contraction (refer to gap allowance temperature chart on page 10). Do not install the H-Trim top cap until all wall panel courses are up.

- Generally speaking, in the colder months, leave a gap of ½” for panels up to 20’ and ¾” for panels over 20’. It is recommended to use the large, faced trims to allow for appropriate expansion and contraction. In the warmer months, leave a gap of 3/8” for panels up to 20’ and ½” for panels over 20’. Refer to gap allowance temperature chart on page 10.

- Installation in weather colder than -10°C (14°F) is not recommended due to potential chipping or cracking of the panels. Should that occur warranty may be voided.

- When installing final top horizontal panel, trimming the full length of the panel at a 45-degree angle may ease installation of last panel. Oversized nail slots must be added manually.

- Steel stud framing, associated sub girts and fasteners to be installed as designated by the appropriate design professional OR must be a minimum of 20 gauge or lower and installed 16-24” O.C.

- Note: DO NOT USE a stapler, or nail gun for exterior cladding and soffit panel installation

- Note: GLUE IS NOT to be used to fasten strapping or ChamClad® exterior wall board

EXTERIOR VERTICAL WALL PANEL INSTALLATION

6” Classic and Shadowline Series

(See assembly drawings listings on page 39)

- To install vertical cladding, position strapping horizontally or diagonally (at a 45° angle) with 12” O.C.  If horizontal, leave 2” gaps between straps every 3’-4’ to allow any envelope moisture to escape.  A continuous, 3-dimensional membrane with a vertical drainage cavity may also be used as a rainscreen (i.e., Homeslicker and follow manufacturers recommended install procedures).  Always check and follow local Building Codes.

- Snap a level chalk line around the base of the building and install universal J Trim/2 pc J Trim that will act as a starter strip, fastening every 8 - 12”. When using a 1 pc Universal J trim, prior to install, drill 3/16” weep holes in bottom of universal J Trim 4” from ends and every 18”-24” to allow for drainage.

- Determine the center of the wall and install 2 starter strips back-to-back (spaced 1 7/8” apart) and 1” above the universal J Trim, fastening every 12”.  Ensure proper vertical backing is behind starter strips.  (See drawing #CD.10 on page 43)

- Continue installing bases of all 2pc. Outside Corners, H-Trims and 2 pc J trims, fastening every 12”. Please note all 2 pc trims require solid backing. We recommend all ChamClad® vertical panels and accessories be fastened every 12”.

- Begin panel installation by positioning first panels into starter strips (working outward from wall center in both directions).  It is good practice to fasten at the highest point of the nail slot at the top of each panel to limit downward migration.  Balance of screws to be fastened in the center of nail slots and should allow for the thickness of a dime (1/32”) between the screw head and the face of the nail slot.

- To complete the install, apply the top caps of all 2pc. trims using a non-marking rubber mallet tapping gently until it locks in place. Do not over drive the trim top cap to the point where the panels can no longer expand and contract due to tightness.

- ChamClad® exterior panels may be used in a post or column wrap application. When installing in a vertical orientation, begin by installing all bases of 2pc Outside Corners on all corners. Continue with installing 1pc Universal J Trim or 2pc J trims between the Outside Corners at the base (punched with weep holes if required) and 2pc J Trim between the corners at the top. Measure column width and trim panels evenly to distribute across the column face with equal panel widths or use two starter strips installed back-to-back vertically in the center of the column and install panels outwards from the center. Fasten all panels at the highest point of the nail slot at the top of each panel to limit downward migration. To complete install, apply top cap of the 2pc outside corner and the 2pc J trim cap along the top. Avoid over hammering the 2pc trim caps so panels can freely move during expansion and contraction. (Refer to vertical column assembly drawings on page 46)

- In addition to using panels in a post, beam, or column wrap application, ChamClad® offers a 7” Column Wrap 2pc. The innovative trim-less design results in clean lines, and an overall modern and precise appearance. It is environmentally friendly and easy to install over any wood or steel post; is extremely durable, UV protected, scratch and graffiti resistant. Refer to detailed installation instructions on pages 35 - 38.

NOTES:

- Attach vertical claddings and trims using ChamClad® Universal Screws. Screws must penetrate structural framing members.

- It is recommended as per CCMC/ ICCES that 6” exterior claddings be installed on an air barrier/ rainscreen. 12” seamless panels must not be used on an exterior wall application. Use on an exterior wall will void warranty on the product.

- Install vertical exterior cladding to structural framing members when using non-structural sheathing, drywall, or foam.  Install exterior cladding a minimum of 2” above finished grade or as per local building codes. Allow 1⁄2” above deck or concrete.

- At every floor level, it is recommended to break the installation through the use of a thru-wall flashing (supplied by others) or a ChamClad® H-Trim and restart following steps above. The first course of vertical cladding ends in the bottom of the H-Trim with the second course starting in the top of the H-Trim following the steps listed above. Always confirm local building code requirements.

- Except where designated in this manual, it is critical that fasteners are installed in the manufacturers pre-drilled holes. If you need to apply a fastener and it is in a section that does not have a manufacturers pre-drilled hole, you MUST pre-drill a nail slot (using a nail slot punch) to allow the panel to expand and contract without restriction.

- Generally speaking, in colder months, leave a gap of ½” for panels up to 20’ and ¾” for panels over 20’. It is recommended to use the large, faced trims to allow for appropriate expansion and contraction. In the warmer months, leave a gap of 3/8” for panels up to 20’ and ½” for panels over 20’. Refer to gap allowance chart on page 10.

- Installation in weather colder than -10 °C (14°F) is not recommended due to potential chipping or cracking of the panels. Should that occur warranty may be voided.

- Note: DO NOT USE a stapler, or nail gun for exterior cladding and soffit panel installation

- Note: GLUE IS NOT to be used to fasten strapping or ChamClad® exterior wall board.

Acclimate all panels and accessories for a minimum of 24 hours prior to installation.

CHAMCLAD® EXTERIOR SOFFIT/FASCIA PRODUCTS

4”, 6”and 12’’ Soffit Panel Series *

6” Solid Soffit Panel 3/8”

4” & 6” Vented Soffit Panel 3/8”

4” Solid Soffit Panel 3/8”

*6” Classic and Shadowline may be used in soffit application (venting supplied by others)

CHAMCLAD® Exterior Soffit Trims

*2PC 1.5” Outside Corner

12’ & 20’

*2pc 1.5” PVC Soffit J Trim

12’ & 20’

1.5” PVC Universal J Trim

12’ & 20’

2pc 2” PVC H-Trim Soffit

12’ & 20’

2pc 2.75” Aluminum H-Trim

12’ & 20’

*THESE TRIMS ARE AVAILABLE IN ALL ALUMINUM, ALL PVC OR HYBRID COMBINATIONS

EXTERIOR SOFFIT INSTALLATION

4”, 6” and 12” Seamless Soffit Panel Series (3/8’’)

(See assembly drawings listings on page 39)

- ChamClad® solid soffit panels are available in 4”, 6” and 12”. The 4” panels are available in 12’ and 20’ lengths, 6” panel in 9’,12’, 14’, 16’, 20’ and 30’. ChamClad® Vented Panel 4” is available in 12’ & 20’ lengths. The 6” Vented panel is available in 12’, 16’ and 20’ lengths. VENTING:  Both vented and solid panels MUST be used during installation of ALL exterior soffit applications. This includes the underside of decks (with or without living spaces above), porch ceilings and typical perimeter soffit for attic ventilation (as required by code).

- 6” X ½” Shadowline may be used in soffit application, non-vent or hidden vent available.

- Verify that soffit substrate is even and level across all surfaces. Applicators may need to install shims to achieve a consistent surface to attach the soffit panels to.

- Attach ChamClad® soffit panels at 12 - 16” using ChamClad® universal screws. If the eave span is greater than 16”, additional blocking must be installed at 12-16” O.C.

- On exterior soffit installations maintain the proper expansion/contraction distance from the end of the board and the required accessory (universal J Trim, 2pc H-Trim or 2pc J trims - refer to refer to gap allowance chart on page 10).

- During the installation process, it is important that the soffit material is installed to freely allow expansion and contraction with changes in temperature. It is recommended to use ChamClad® universal screws. Please allow for a 1/32” gap from the fastener head to the nail flange. If installed without this allowance, ChamClad® soffit panels may warp or buckle. Due to our PVC’s recycled composition, ChamClad® soffit will not swell if exposed to moisture.

When installing vented soffit panels, it is recommended to alternate between solid and vented panels.  Always begin installation with a solid soffit panel. First panel, you must rip down nose to tuck inside the 2 pc J trims and allow for a gap (refer to gap chart page 10). Do not forcefully press panels together during installation to avoid cupping; panels must be installed loosely to allow for movement. Do not over torque screws when installing soffit.  When installing long lengths install from one side working your way down to avoid buckling. Confirm requirements with local building code.

Note: All 2pc trims require solid backing. All soffit applications require trims to be      fastened at 8-12” O.C.

- Depending on the design of the soffit, ChamClad® soffit panels may be installed parallel or perpendicular to the exterior walls. It is important that each panel be fastened at every joist or rafter every 12”-16” O.C. using ChamClad® Universal Screws. Install additional courses and check alignment to either side of the line. Check that proper gaps according to the typical weather conditions are maintained to allow for proper expansion/contraction.

When installing vented soffit panels, it is recommended to alternate between solid and vented panels.  Always begin installation with a solid soffit panel. First panel, you must ripped down nose to tuck inside the 2 pc J- trims and allow a ¼” gap. Do not forcefully press panels together during installation to avoid cupping; panels must be installed loosely to allow for movement. Do not over torque screws when installing soffit.  When

Soffit – Parallel to Building

installing in long lengths install from one side working your way down to avoid buckling. Confirm requirements with local building codes.

Soffit – Perpendicular to Building

When Wrapping Fascia through soffit, install Outside Corner horizontally along the bottom of the fascia. The top portion serves as a starter for the fascia with the underside portion being the starter for the soffit

- For both 6” and 4” x 12’ vented soffit panels, the ventilation area is 13.25 in2 per panel; for 20’ vented soffit panels, the ventilation area is 22.08 in2.  ALWAYS reference local building code when calculating ventilation requirements.

- ChamClad® does not recommend installing products at -10°C (14°F)

- Allow the product to be acclimated for a minimum of 24 hours before install. Generally speaking, in colder months, leave a gap of ½” for panels up to 20’ and ¾” for panels over 20’. It is recommended to use the large, faced trims to allow for appropriate expansion and contraction. In the warmer months, leave a gap of 3/8” for panels up to 20’ and ½” for panels over 20’. Refer to gap allowance chart on page 10.

- Where more than one soffit length is needed, an H-Trim is required to cover the expansion/contraction gap.  DO NOT BUTT JOIN PANELS.

Please note 2pc trims require solid backing and be fastened every 8-12” O.C.

- When installing Trims and Soffit panels, it is important to use low profile ChamClad® universal screws and fasten solid soffit and vented panels every 12”- 16” and all trims fastened every 8”-12”.

In high wind or heat areas all soffit panels must be fastened 12” O.C.

- When installing final panel, trimming excess panel at a 45° angle at the full length of the panel may ease installation of last panel. Nail slots must be added manually.

NOTE:   4” and 6” soffit panels may NOT be used in exterior WALL application as improper use will VOID WARRANTY.

DO NOT install gas heaters into a soffit or ceiling application.

If planning to install infrared heaters, check with the Chamclad® engineering department to ensure compatibility with cladding.

INTERIOR WALL AND CEILING CLADDING

4” & 6” V GROOVE/U CHANNEL AND 12” SEAMLESS PANELS

12” Interior Wall & Ceiling Panels Seamless Panel 3/8”

6” Interior Wall & Ceiling Panels 3/8”

4” Interior Wall & Ceiling Panels 3/8”

NOTE:  4” & 6” panels available in V-Groove and U-Channel profiles

*6” Classic and Shadowline may be used in interior wall and ceiling applications

Interior Wall and Ceiling PVC Trims

2pc 1.5” PVC J Trim

12’ & 20’

2pc 1.5” PVC Outside Corner Trim

12’ & 20’

1pc PVC Inside Corner Trim

10’, 12’, & 20’

1.5” PVC Universal J Trim

12’ & 20’

2pc 2” PVC H-Trim

12’ & 20’

COLUMN AND BEAM WRAP

7” 2pc Column Wrap

Quick Click Column Cap and Base Trim

Available in 7”, 8”, 10”, and 12”

1” X 4” Architectural Trim

Beam End Cap

2pc Beam 5.5”x 3.35”

Beam End Bracket

Quick Click Column Cap and Base Trim

Available in 7”, 8”, 10”, and 12”

4pc Quick Click Column and Beam Wrap

Available in 8”, 10”, and 12”

New Summer 2024

HORIZONTAL INTERIOR INSTALLATION

4” & 6” VGroove/UChannel and 12” Seamless Panels

(See assembly drawings listings on page 39)

- ChamClad® Interior Horizontal panels are installed similarly to Exterior Horizonal panels; please refer to Exterior Horizontal Guidelines on pages 21 – 22.

- Always allow ChamClad® panels and accessories to acclimatize to current temperature conditions before starting the installation. Temperature variations are less on an interior installation.

- Verify that the walls and ceiling substrates are even and level across all surfaces. Furring may be required to acquire a consistent surface to attach ChamClad® panels.

- Establish a level guideline using a chalk line on all wall areas to be covered with ChamClad® wall cladding, where the bottom of the lowest board is to be installed. Depending on the look or finish that is desired, the installer may use a starter strip or a universal J Trim. Since this is an interior wall, no drain holes need be drilled into the bottom of the universal J Trim.

- Install universal J Trim or 2pc J Trim in areas where ChamClad® wall panels meet other finishes. Also install Universal J Trim or 2pc J Trim at all windows, doors, and other penetrations. It is common practice to fasten interior wall panels at 16- 24” O.C. Every few courses of wall panels, the installer should check for horizontal alignment and level.

- Where the interior wall meets the ceiling, and depending on design requirements, use the ChamClad® 2pc J Trim. Install all trim bases first and once ChamClad® wall panels are installed to the top course, finish off by installing the top caps.

- When joining ChamClad® interior wall panels, use 2pc H-Trim to connect panels. It is recommended to align the ChamClad® 2pc H-Trim with other trim elements on the wall.

- On Interior applications, fasten trims (12-16” O.C.) and panels (16–24” O.C.) using ChamClad® universal screws. Panels must be attached to structural framing members.

DO NOT USE STAPLERS/PNEUMATIC STAPLERS FOR INTERIOR OR EXTERIOR APPLICATIONS.

*DO NOT USE 4” and 6” INTERIOR panels in exterior applications.  The use of 4”,6” and 12” Interior wall panels on an exterior wall will void the warranty.

INTERIOR VERTICAL INSTALLATION

4” & 6” VGroove/UChannel and 12” Seamless Panels

- ChamClad® Interior Vertical panels are installed similarly to Exterior Vertical panels; please refer to detailed Exterior Vertical instructions on page 23.

- Always allow ChamClad® panels and accessories to acclimatize to current temperature conditions before starting the installation. Temperature variations are less on an interior installation, but the same requirement applies.

- Verify that the walls and ceiling substrates are even and level across all surfaces. Furring may be required to acquire a consistent surface to attach ChamClad® panels.

- Establish a level guideline using a chalk line on all wall areas to be covered with ChamClad® wall cladding, where the bottom of the lowest board is to be installed.  As in the Exterior Vertical Installation, use a universal J Trim to start application. As this is an interior wall, no drain holes are required to be drilled into the bottom of the universal J Trim.

- Commence application with installation of all trims at 12-16” O.C.

- When covering entire wall area, install starter at mid-point of wall and align VGroove or UChannel to the centre of the wall and work outwards from either side. Install balance of trim bases as per exterior vertical installation and finish wall application with installation of 2pc trim caps. It is good practice to fasten at the highest point of nail slot at the top of each panel to limit downward migration.

- If applying interior vertical panels as a feature on a portion of a wall area, use full panels (cut to height) in order to maximize overall appearance.  Begin with installation of component pieces and base trims; followed by installation of all wall panels (working left to right) and complete with application of component top caps. Attach panels to structural framing members.

DO NOT USE Electric Staplers/Pneumatic Staplers for INTERIOR APPLICATIONS.

We recommend using Chamclad® Universal Screws but if unavailable use a non-corrosive Low Profile #8 Modified Truss Head Screw 1 ¼”.

DO NOT USE 4”, 6” and 12” INTERIOR panels in exterior wall applications.  The use of 4”,6” and 12” Interior wall panels on an exterior wall will void the warranty.

INTERIOR CEILING INSTALLATION

4” & 6” VGroove/UChannel, 12” Seamless Panels and Faux Beam Wrap

(See assembly drawings listings on page 39)

- ChamClad® interior wall panels may also be used as a ceiling finish; installed similar to ChamClad® exterior soffit panels - refer to page 25.

- Always allow ChamClad® panels and accessories to acclimatize to current temperature conditions before starting the installation. Temperature variations are less on an interior installation, but the same requirement applies.

- Verify that ceiling substrates are even and level across all surfaces. Furring may be required to achieve a consistent surface to attach ChamClad® panels. Install furring strips (1x3”) at 12-16” O.C. perpendicular to the joist.

- We recommend all ChamClad® accessories be fastened every 12-16”

- Fasten at each furring strip, which should be 12-16” O.C. Once you have installed a few courses of ChamClad® ceiling panels, check for alignment of the perpendicular or parallel adjacent vertical wall. Where you are placing panels end to end, use H-Trim to finish the joint.

- Install the remaining ChamClad® panels and check for alignment. When using the 2pc J Trim install the top cap onto the base to complete the installation.

Note:

- ChamClad® versatile 4”,6” & 12” interior architectural panels are designed for all types and sizes of decorative (non-load bearing) post, beam and column wrap applications. In addition, we offer a 2pc Faux beam wrap which is lightweight and easy to install. Please refer to detailed installation instructions on page 38.

  - 4”, 6”, and 12” interior 3/8”panels must be used for soffit applications, interior covered deck areas, barrel ceilings, sun porches, interior walls, and ceilings. The use of these panels on an exterior WALL will void the warranty.

ChamClad® 6” Ceiling Panels        (SunBleached Oak)

COLUMN AND BEAM WRAP INSTALLATION

7” 2pc Column Wrap and 4pc Quick Click Column and Beam Wrap

(See assembly drawings listings on page 39)

1. LOADING
o An internal structural support must be used if the column is for non-load bearing or for load bearing applications.

2. DIMENSIONS
o Exterior dimensions of the column are 7” x 7”.
o Internal cleared dimensions of the column are 6” x 6-1/2”.

o Architectural trim dimensions are 3/4” x 3-5/8”.
o Column lengths available in pairs at 10ft and 12ft lengths.
o Architectural trims available at 6.5ft and 13ft.

3. INSTALLATION
o The internal structural support should be straight, square, and within the dimensions specified above.
o Cut 2-piece columns caps 1/4” shorter than column height requirements to allow for expansion at the top of the column. (1/8” for winter installation).
o Wrap the 2-piece columns around the vertical internal structural support by engaging the arrowhead in one panel into the groove of the second panel. Do this on both ends of the “L” shaped column, using a non-marking rubber mallet to assemble. Ensure a tight wrap around the internal structure to avoid movement and full engagement of the two pieces for a seamless finish.

o Fasten column against substrate support on bottom only where trims would hide the fastener. Ensure fastener is straight and flush into PVC. Do not fix PVC column with nails or screws on the top of the column to allow for expansion and contraction at the top of the column.

Optional:
If column is loose or has movement around the internal structural support: Create three collars measuring 6-1/2” x 5-1/2” around the base, center, and top of the support structure. Install the 2pc column, which will form a tight fit around the collars. This will prevent movement of the 2pc column.

4. FINISHING

Option A – Architectural Trims

o ChamClad® offers architectural trims to finish the ends of the columns.
o Cut architectural trims at 45-degrees. Ensure the trim is pressed firmly against the column when determining the edge of the 45-degree cut. Cut 2 sets to install on top and bottom of the column.

o Touch up exposed cut edges with touch-up paint. Paint cans or touch up pens available through ChamClad®.

o Position trims with film facing up as per above and apply one continuous strip of masking tape. Flip the trims over and apply silicon on 3 of the 4 trims on the unlaminated face.

o Apply additional silicon if needed at the seam between the trim and the column.
o Fold over the excess masking tape on trim 4 to hold the trims in place firmly against the column wrap.
o Use 1-1/2” finishing nails to close gaps at mitered corners of trims. Do not fasten trims into PVC column.
o Duplicate the above instructions for the trim on the top of the column and line up trim to the ends of the column. You should see a 1/8 - 1/4” gap between trim and ceiling to allow for column expansion.
o Once silicon cures, remove masking tape and touch up finishing nail heads to blend in with the film finish.

Option B – Quick Click Column Cap and Base Trim

o Install 2pc column as per previous instructions

o Attach 4pc column trims snug around column and slide them together until they lock in place

o Attach the 4 ground fasteners at each corner,  3/16 X 1 ¼ Hex Concrete Screw (4 per box) and #10 X 1 ¼ flat head for concrete (4 per box)  provided in kit.

o Attach side fasteners #8 pan socket type S zinc plated (4 per box) provided in kit, into the column through the side of base clip  (only fasten at base leave top loose for expansion and contraction)

o Snap each of the 4 covers over the base, you may touch      up the ends of covers before install if desired.

O Repeat for trim on cap if required, “not” attaching side fastener.

Quick Click Column and Beam                                                                                     New Summer 2024                                                                                                                       Available in 8”, 10”, 12”, Mix and Match

Unassembled

10”

8”

Assembled

Unassembled

Assembled

Scan QR Code for Updates on Quick Click Column and Beam

2PC FAUX BEAM INSTALLATION                                                     (See assembly drawings listings on page 39)

1. LOADING
o ChamClad® 2pc Beam System is not load bearing and should not be used as primary source of weight support.
o Internal Weight Capacity of Beam = 24.11 kg/m (16.17 lbs/ft) (equally distributed weight). o Weight of Beam and Adapter = 1.84 kg/m (1.23 lbs/ft)

2. DIMENSIONS
o Exterior dimensions of the beam are 5-1/2” x 3-3/8”
o Internal cleared dimensions of the column are 43⁄4” x 3”. 
o Beam cap and Adapter lengths available in pairs at 10ft, 16ft, and 20ft lengths.

3. PACKAGING
o Beam cap and Adapter will be packaged together in vented white bags, one set per bag.

4. INSTALLATION
o ChamClad ® 2pc beam is compatible for interior applications.
o Allow 2pc beam system to climatize to installation temperatures prior to cutting. Measure and mark beam length using a square edge.
o If fitting against an angled wall or ceiling, a T-bevel may be helpful to determine the cut angle.
o Use a fine-toothed circular saw to cut beam cap and adapter. Ensure equal lengths of both adapter and beam are cut.
o Mark ceiling with a chalk line to ensure a straight installation of the beam adapter.
o Fasten beam adapter using ChamClad® universal screws every 16-24”. Ensure fasteners are fastened into a joist or a stud. If no solid backing is available, use drywall anchors suitable for the weight of the beam cut (install as per anchor manufacturer instructions). See LOADING above for weight of beam and adapter. Fasten adapter using all 4 screw grooves available.

o Add ChamClad® beam bracket along wall to close any spacing between wall and beam good for renovation (2 per pack screws, #8 1 ½” flat quadrex course thread plated black zinc) 8 per bag.

UNASSEMBLED

ASSEMBLED

BEAM END CAP

BEAM END BRACKET

ASSEMBLY DRAWINGS

HORIZONTAL EXTERIOR

  - see drawings CD.3 & CD.5 (pages 40 & 41 )

VERTICAL EXTERIOR

  - see drawings CD.8 & CD.10 (pages 42 & 43)

SOFFIT DETAILS

  - see drawing CD.15 (page 44)

INTERIOR HORIZONTAL

  - see drawings CD.5  (page 41)

INTERIOR CEILING

  - see drawing CD.13 (page 51)

COLUMNS & BEAMS USING CHAMCLAD CLASSIC OR SHADOWLINE PANELS

  - see drawings CD.19, CD20, CD.16 (pages 45, 46 & 47)

7” COLUMN WRAP 2PC QUICKCLICK SYSTEM

  - see drawing CD.27 AND CD.28 (pages 48 – 49)

2PC FAUX BEAM QUICKCLICK SYSTEM

  - see drawing CD.18 (page 50)

If required use touch up pen on laminated snap covers

If required use touch up pen on architectural trim mitre cuts

QUICK CLICK COLUMN CAP/BASE (EXPLODED DETAIL)

Install the beam cap and fasten the object directly onto the beam cap and into the wood

Solid Soffit

Care and Maintenance of ChamClad® Exterior Wall Panels

ChamClad® products are designed to be low maintenance and our Laminate Films have exceptional properties that ensure ease of cleaning and surface longevity.  They are highly ultra-violet, scratch, and salt air resistant.  The film’s low surface tension makes it inherently dirt-repellent and therefore easy to clean.  ChamClad® finishes include anti-graffiti properties and, in most cases can be easily removed using a garden hose and mild kitchen detergent and warm water.

If cleaning required, follow these simple instructions below:

  - Using a soft bristled nylon brush attached to a garden hose, clean the surface with a mild PH neutral soap or non-abrasive kitchen detergent in warm water. DO NOT USE abrasive, erosive, or caustic solvents to clean ChamClad® panels as they can damage the panel finish and will void the warranty.

  - To avoid streaking start at the top of the wall – working your way to the bottom.

  - Follow with a thorough rinsing of fresh water to ensure that substances such as airborne salts, dirt and pollutants are removed.

  - Always remove unintentional paint splashes, stucco spills, etc. immediately to prevent the possibility of permanent staining.

  - You may pressure wash ChamClad® panels with a low PSI pressure washer. DO NOT exceed 1500 PSI.

Should you experience removal difficulties, please contact us via email at info@ChamClad.com.

| Installation Tips - At a Glance. Before You Start | Do Review the installation manual and all local building codes before starting the install. | Don’t Don’t install damaged or questionable materials. Check all products and call ChamClad® customer service. |
| --- | --- | --- |
| - Storage and Handling | - If storing cladding outside, when possible, keep out of direct sunlight. - Store on a flat level surface, cover with a tarp leaving the ends open for air circulation. - Carry ChamClad® panels by holding it by the narrow edge and pick panels up in the center. | - Don’t allow panels to come in contact with asphalt or in areas prone to excessive heat buildup. - Don’t use abrasive cleaners or solvents to clean ChamClad® cladding. |
| - Flashing | - Use non-corrosive flashing. - Always leave a minimum of ¼” gap between the bottom of the cladding and metal flashings | - Don’t use uncoated aluminum or other metal materials. |
| - Wraps and Sheathing | - Install horizontal and vertical exterior cladding to structural framing when using non-structural sheathing, drywall, or foam. - Cladding must be installed onto structural framing/sheathing and over a water-resistant barrier in accordance with local building codes. | - Don’t fasten ChamClad® cladding over non-structural framing or sheathing (e.g., foam) without re-establishing a structural surface like a rain screen or furring strips. |

| - Clearance | - Install exterior cladding at least 2” or as per building code requirements above finished grade. Allow ½” above deck or concrete. - Refer to the ChamClad® Gap allowance temperature guidelines on page 10 in order to allow for proper expansion/contraction between panels and trims. |  |
| --- | --- | --- |
| - Cutting | - Carbide-tipped blades with 100+ teeth are recommended. (Aluminum blades are optimal) - If using a circular saw, cut cladding face down on a clean surface. - If using a cut-off saw, cut face up. | - Don’t score or snap ChamClad® exterior cladding. - Don’t cut ChamClad® with a coarse saw blade to avoid chipping or damaging the panel. |
| - Butt Ends | - If attaching ChamClad® cladding end to end, use H-trim and cut panels to the appropriate length to allow for expansion and contraction. | - DO NOT butt join. - Don’t place panels into H-Trim without proper gap for expansion and contraction (refer to chart on page 10). - Do not leave gaps between two butt ends without utilizing an H-trim. Do not glue butt ends. |
| - Fastening | - Screws must penetrate the structural framing members. - Fastener must be installed into the center of the nailing slot. - Fasteners must penetrate a minimum of 1 ¼” into structural framing. Structural sheathing is a minimum of 7/16” OSB or plywood (as per local building codes). - Use Chamclad® Universal screws; stainless fasteners or corrosion resistant are recommended (e.g., 17 Self taping, round washer, lo root #8) - Always leave a 1/32” gap (width of a dime) between the fastener head and nailing slot | - Don’t counter sink or angle the fastener. PVC cladding must be hung and be allowed to expand and contract freely. - Don’t use galvanized screws. - Don’t use soffit staples, D-head nails, finishing nails or glue to install exterior. - Don’t fasten panel into the bottom of trims. - Don’t exceed recommended spacing between fasteners. |
| Additional Tips: | - Use ChamClad® touch-up pens to cover any exposed PVC substrate resulting from cutting or a surface scratch. - Use backing for all trims. - Fasten panels 16-24” OC. - Fasten trims 8-12” OC. - Fasten Soffit at 12-16” OC. - Use centre of nail slots. - Cut panels shorter to allow for expansion/contraction clearance. See Gap Chart on page 10. - Use a non-marking Rubber mallet or hand pressure to install V-groove system and top caps of all 2pc trims. - Break vertical installation at every floor level using ChamClad® H-Trim or flashing detail (supplied by others). | - DO NOT install gas heaters into a soffit or ceiling application! - If planning to install infrared heaters, check with the Chamclad® engineering department to ensure compatibility with cladding. |

Glossary

This guide is written for construction professionals. Language typical of the cladding and cladding industry has been used throughout this document, the majority of which has been defined below. A simplified guide is available at the end of this document.

Back Sealing/Priming: Applying a sealer/primer to the back of cladding is NOT recommended for use with ChamClad® heavy gauge panels.

Belly Band: A decorative horizontal trim piece or board placed between two floors or types of cladding, usually along the rim joist.

Beaded: A narrow, half-round molding at the base of a cladding panel or board.

Blind Nailing: Fastening through the top edge of a lap cladding to hide the fastener head with the next course of cladding.

Butt End: A joint created by placing adjoining edges of cladding end to end with no overlap.

Casing: Molding of various widths used to trim around windows and door openings at the jambs. These are also known as lineal window and door surrounds.

Contraction: This commonly is referred to a building material contracting due to heat loss or moisture leaving the product. Generally, this occurs when the outside temperature drops.

Course: A row of cladding running the width of a house or wall. In the case of vertical siding, it would run the height of the wall.

Drip Cap: A horizontal flashing placed over an exterior door or window to divert rainwater away from the wall.

Eave: The overhang of a pitched roof; usually made up of a fascia board, a soffit for closed cornices, and the necessary trims.

Expansion: Most often refers to building materials expanding due to outside temperature increases. In the case of wood or wood by-products, expansion may be caused by moisture entering the product.

Exposure: The width of the exposed face of the board, sometimes called the reveal.

Face: The exposed side of the cladding, trim, or soffit after the material is installed.

Face Nailing: This is where a nail is fastened through the overlap; this exposes the head of the nail, screw, or staple. This is NOT a recommended method of fastening ChamClad® cladding and soffit panels.

Fascia: The flat, horizontal band that covers the rafter tails at the bottom edge of the roofline.

Flashing: A material (usually metal) placed around openings to prevent water penetration or to direct the flow of rainwater over the cladding.

F- Channel Trim: The horizontal trim used to connect and support the soffit at the top of the cladding.

Furring Strip: Long, thin lengths of wood or metal strips used to build out the surface of the wall to facilitate fastening. It is also used to even out imperfections on the wall surface, establish a rain screen, or provide a structural fastening surface on non-structural materials like rigid foam insulation.

Gable: A triangle formed by a sloped roof on the side or front of a house.

Hot Dipped Galvanized: A process whereby metal is dipped into molten zinc to apply a protective coating to prevent corrosion.

H-Channel Trim: Used to conceal cut edges when installing horizontal and vertical cladding or soffit material. The trim covers butt ends or joints as they approach each other. This is mainly for aesthetic purposes.

Lap cladding: Where two pieces of cladding over-lap each other.

Lineal: Moldings of various widths used to trim door and window openings at the jambs. Also known as casings or window/door surrounds.

Mitre Cut: A beveled cut (usually 45 degrees) made at the end of a trim or board, creating a mitre joint.

O.C.: On centre. The distances between the center of two members in a structure, most often studs.

Profile: The contour or outline of a cladding board, viewed from the side.

Rain-screen Wall: This is a method for building walls where the cladding is separated from a membrane by airspace. This allows the equalization of the air pressure behind the cladding and prevents rainwater from being forced in. This is usually created by using furring strips behind the cladding and over the weather-resistant barrier.

Rigid Sheathing: Plywood, OSB, or foam sheathing.

Rim Joist: A board that the rest of the joists are nailed to that run the entire perimeter of the house.

Rip-Cut: A cut made lengthwise on a board or plank along the grain.

Shim: A building material (usually made of wood) used to even a surface.

Soffit: The underside of an overhanging eave. Depending on local building codes, this can be vented or non-vented.

Square: A unit of measure for cladding or roofing equal to 100 square feet of exposure (e.g., 10’ x 10’ = 100 square feet = 1 “square”).

Starter Strip: A trim piece used to start the first course (row) of cladding. This ensures a consistent visual line.

Structural Sheathing: A layer of boards, wood, or other materials applied to the outer studs, joists, or rafters to strengthen the structure and serve as a stable base for the exterior cladding.

Substrate: A layer of material applied to the exterior walls of a building.

Weather-Resistant Barrier: A membrane that protects the exterior wall envelope from exterior wind and water penetration.

Sustainable wood solutions for today’s world.

Revised: March 2024

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 1

SPEC NOTE:  This Section specifies a rigid PVC interlocking siding with a factory laminated PVC film, suitable for residential and commercial exterior wall applications.

DISCLAIMER:  Users of this guide specification are responsible for determining its suitability for a particular project and for all necessary editing to suit. Chameleon Wrapping & Lamination makes no representations or warranties with respect to its accuracy or completeness and disclaims any implied warranties of merchantability or fitness for a particular purpose and shall not be liable for any loss of profit or any incidental, consequential or other damages arising from its use.

# General

## SECTION INCLUDES

#### This Section includes requirements for supply of rigid polyvinylchloride (PVC) interlocking siding boards with laminated PVC film on exposed surfaces and its installation on exterior walls.

## RELATED REQUIREMENTS

#### Section [07 25 00 - Weather Barriers].

#### Section [07 60 00 – Flashing and Sheet Metal].

#### Section [07 92 00 - Joint Sealants].

## REFERENCES

#### CCMC Registry of Product Evaluations.

#### CAN/CGSB-41.24-95, Rigid Vinyl Siding, Soffits and Facia.

#### CAN/ULC-S101-14 Standard Methods of Fire Endurance Tests of Building Construction Materials.

#### CAN/ULC-S102-2018, Standard Method of Test for Surface Burning Characteristics of Building Materials and Assemblies.

#### CAN/ULC-S102.2-2018, Standard Method of Test for Surface Burning Characteristics of Flooring, Floor Coverings, and Miscellaneous Materials and Assemblies.

#### CAN/ULC-S134-13-REV1, Standard Method of Fire Tests of Exterior Wall Assemblies.

#### ASTM D3679-17, Standard Specification for Rigid PVC Siding.

#### ASTM E119 -22, Standard Test Methods for Fire Tests of Building Construction and Materials.

#### ASTM E330/E330M-14(2021), Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference.

#### ASTM G154- 16, Standard Practice for Operating Fluorescent Ultraviolet (UV) Lamp Apparatus for Exposure of Nonmetallic Materials.

#### ASTM G155-21, Standard Practice for Operating Xenon Arc Lamp Apparatus for Exposure of Materials.

#### ASTM D635-18, Standard Test Method for Rate of Burning and/or Extent and Time of Burning of Plastics in a Horizontal Position.

#### American Architectural Manufacturers Association (AAMA):

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 2

#### AAMA 303-19, Voluntary Specification for Rigid Polyvinyl Chloride (PVC) Exterior Profiles.

#### AAMA 307-16, Voluntary Specification for Laminates Intended for Use on AAMA Certified Profiles.

#### AAMA 664-21, Voluntary Specification for In-Process Quality Control Requirements for Applicators of Synthetic Interior and Exterior Laminates.

## COORDINATION

#### Coordinate work of this Section with installation of flashings to provide drainage to exterior.

## ACTION SUBMITTALS

#### Submit proof of manufacturer’s CCMC listing and listing number.

#### Product data: Submit manufacturer's printed product literature, specifications and data sheets in accordance with Section [01 33 00 - Submittal Procedures] [___].

#### Samples:

##### Submit samples [in accordance with Section [01 33 00 - Submittal Procedures] [___].

##### [Submit colour samples for colour/pattern selection]. Submit duplicate samples of actual siding material, full profile width x [200] [___] mm long, of specified colour/pattern and profile.

##### [Submit samples of all exposed accessories and trims].

#### Installation Instructions: Submit manufacturer's printed installation instructions.

#### Installer Qualifications: Submit proof of installer compliance with installer qualifications specified under Quality Assurance.

#### Sustainable Design Submittals: Submit required documentation in accordance with Project sustainable design requirements in Section [01 35 64 – LEED Requirements] [01 35 67 – Sustainability Certification Requirements] [___].

## CLOSEOUT SUBMITTALS

#### Submit in accordance with Section [01 78 00 – Closeout Submittals] [___].

#### Maintenance Data: Submit manufacturer's printed instructions for cleaning and repair procedures. Include names and contact information of Product manufacturer and installer.

#### Warranty Documentation: Submit manufacturer’s warranty.

## MAINTENANCE MATERIAL SUBMITTALS

#### Submit in accordance with Section [01 78 00 – Closeout Submittals] [___].

#### Supply:

##### One manufacturer’s touch-up pen for each colour of siding material.

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 3

##### [1%] [__%] of installed boards in manufacturer’s maximum standard board length.

##### [One] [___] of each accessory and trim shape in manufacturer’s maximum standard length.

#### Package materials with protective coverings and label.

## QUALITY ASSURANCE

#### Comply with AAMA 664-21.

#### Test Reports: Submit certified test reports showing Product compliance with specified performance characteristics and physical properties.

#### Installer Qualifications: Minimum [one] [three] [___] year[s] experience installing specified Products or similar products, on projects of similar size to this Project.

#### Pre-Installation Meeting: Conduct pre-installation meeting to verify installation requirements, manufacturer's installation instructions, and manufacturer's warranty requirements.

#### Mock-Up: In accordance with Section [01 40 90 – Quality Requirements] [___].

##### Construct a [___] mm x [___] mm mock-up of siding installation [not] forming part of the Work for Consultant’s review.

##### Include each accessory and trim type, [an [outside] [inside] corner] [an intersection with [adjacent material] [a door] [a window]].

##### Approved mock-up establishes an acceptable standard for the Work.

## DELIVERY, STORAGE, AND HANDLING

#### Comply with manufacturer’s instructions.

## SITE CONDITIONS

#### Proceed with installation only when ambient temperature is above -20 C.

## WARRANTY

#### Provide the following manufacturer’s limited warranty coverages:

##### That Product will be free from initial manufacturing defects in material or workmanship and against warping, blistering, and peeling for 50 years, prorated.

##### Against unreasonable discolouration for 15 years, non-prorated.

# Products

## MANUFACTURER

#### ChamClad, 10235 – 184 St., Edmonton, AB, T5S 2J4. chamclad.com Ph. 780-454-4430.

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 4

#### [Substitutions will not be considered.] [Substitutions will be considered during bidding period as specified in Section [00 21 13 – Instructions to Bidders] [___].] [Substitutions will be considered as specified in Section [01 25 00 – Substitution Procedures] [___].]

## MATERIALS

#### PVC Extrusions: Engineered, rigid, PVC extrusions with 100% pre-consumer recycled content, to AAMA 303-19.

#### Rigid, interlocking, PVC panels with hollow core profile and factory laminated PVC film, to CAN/CGSB-41.24-95 and ASTM D3679-17.

#### Classic Board Profile: Nominally 150 mm wide x 13 mm thick.

#### Shadowline Board Profile: [Nominally 150 mm wide x 13 mm thick with [150 mm] [75 mm reveal.] [Nominally 300 mm wide x 13 mm thick with 50 mm reveal.]

#### Board Lengths: [maximum standard lengths to minimize joints] [custom lengths to minimize joints and minimize waste].

#### Colour/pattern: [Barn Board Grey] [Beach Wood Grey] [Black Sand] [Black Wood] [Barrel Oak] [Brown Oak] [Brushed Metallic] [Brushed Silver] [Canadian Pine] [Charred Black] [Cinnamon Walnut] [Smoke Grey] [Spanish Cedar] [Speckled Graphite] [Sun Bleached Oak] [Teak] [Bamboo] [Toffee] [Black Walnut] [California Redwood] [Chocolate Walnut] [Atlantic White] [Chai Cedar] [Desert Oak] [Golden Oak] [Brushed Titanium] [Honey] [Iron Glimmer White] [Modern Walnut] [Monument Oak] [Mountain Oak] [Mountain Pine] [Natural Oak] [Navy] [Pure White] [Toffee] [Tofino] [Dark Wood] [Harvest Walnut] [Brushed Metallic] [Crystal White] [Warm Autumn] [Pebble Brook] [Sand Dollar] [Grey Asphalt].

#### Accessories and Trims: [PVC] [Aluminum] starter strip, aluminum/PVC two-piece J-trim, [aluminum/PVC two-piece outside corner] [aluminum H Mold] [PVC one piece J-trim] [PVC inside corner], matching finish to siding, pre-punched nailing holes, [maximum standard lengths to minimize joints] [custom lengths to minimize joints and minimize waste].

#### Furring: [____mm] x [____mm] [pressure treated wood furring strips] [galvanized steel] [aluminum] [furring channels] [as indicated on Drawings].

#### Fasteners: [galvanized] [stainless] steel.

## PERFORMANCE CRITERIA

#### Material Characteristics: to CAN/CGSB-41.24-95.

#### Dimensional Tolerances: to CCMC TG 07 46 33.07.

#### Flammability: Category HB when tested to ASTM D635-18.

#### Acid Rain Resistance: to CCMC TG 07 46 33.07.

#### Wind Load Resistance: to CCMC TG 07 46 33.07 and to ASTM E330/E330M-14(2021).

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 5

#### Durability: to CCMC TG 07 46 33.07.

#### Accelerated Weathering Resistance: Meets ASTM G155-21 and ASTM G154-16. Meets ASTM D3679 for weathering after 24 month exposure.

#### Surface Burning Characteristics: Flame spread rating < 75 and smoke developed classification <450 when tested to CAN/ULC-S102-2018. Flame spread rating < 120 when tested to CAN/ULC-S102.2-2018.

#### Flammability: Category HB when tested to ASTM D635-18.

#### Fire Test of Wall Assembly: Meets CAN/ULC-S101-14, ASTM E119, and CAN/ULC-S134-13.

#### PVC Film: to AAMA 307-16.

# Execution

## VERIFICATION OF CONDITIONS.

#### Verify that substrates are suitable for installation of the Work of this Section. Commencement of Work denotes acceptance of conditions.

## INSTALLATION

#### Install furring [strips] [channels] to provide rainscreen.

#### [Install Classic siding [horizontally] [vertically] [diagonally] [as indicated on Drawings] for [U-Channel] [V-Groove] appearance.]

#### [Install Shadowline siding [horizontally] [vertically] [diagonally] [with mix and match of different profiles] [as indicated on Drawings].]

#### Install siding, accessories and trim in strict accordance with manufacturer’s printed instructions.

#### Scribe and cut as required to neatly fit abutting walls and adjacent surfaces, and to accommodate other materials penetrating through boards.

#### Secure boards, accessories, and trim in place, rigid, plumb and square.

## ADJUSTING

#### Where minor surface damage has occurred, touch-up by methods recommended by manufacturer. Remove and replace damaged materials that cannot be satisfactorily repaired.

## CLEANING

#### Upon completion of installation, remove surplus materials, rubbish, tools and equipment, and clean as specified in Section 01 74 00 – Cleaning and Waste Management.

ChamClad	                                                                                                                           Section 07 46 33

Guide Specification	                                                                                  LAMINATED FILM PVC SIDING

Feb. 13, 2024	                                                                                                                                     Page 6

#### Where necessary, remove dirt and other contaminants in accordance with manufacturer’s cleaning instructions.

END OF SECTION

Warranty:

## COVERAGE UNDER THIS WARRANTY

Chameleon Wrapping and Lamination Ltd. is pleased to offer a 50-Year Limited Warranty (15 years non pro-rated) on its ChamClad® line of Rigid PVC siding, cladding and accessories (the “product”), on the terms and conditions set out herein (the “Warranty”).

Chameleon warrants the product will be free from initial manufacturing defects in material or workmanship (“initial defects”) for a period of 50 years (15 years non pro-rated) from the date of purchase. Provided the product has been stored and installed according to ChamClad® installation instructions, the product is warranted for a period of 50 years (15 years non pro-rated) (as prorated as set out below) against warping, blistering, and peeling (“technical failure”), subject to the limitations contained herein.

The product, like all materials, will experience some degree of discolouration due to age, sun exposure, and other environmental conditions. Provided the product has been stored and installed according to ChamClad® installation instructions, Chameleon further warrants the product against unreasonable discolouration for a period of 15 years from the date of purchase (subject to the limitations contained herein). In the event a claim is brought under the Warranty for unreasonable discolouration, Chameleon, in its sole discretion, will determine whether the degree of discolouration or fading is unreasonable or excessive, having regard to the factors set out below.

In the event of initial defects, technical failure or unreasonable discolouration covered by the Warranty, Chameleon will, at its sole discretion, repair, restore or replace the product in accordance with the claims provisions set out below. Chameleon’s responsibility is limited to the product only. Coverage for installation and labour costs will not be covered by Chameleon at Chameleon’s sole discretion, and unless otherwise agreed by Chameleon in writing, all labour and installation costs are the sole responsibility of the homeowner. The warranty on the repair or replacement product will be limited to the unexpired term of the Warranty for the product which was repaired or replaced.

The Warranty extends to the original purchaser(s) of the product. The Warranty shall terminate upon the sale of the property or the death of the original purchaser(s)/homeowner(s), unless the Warranty is transferred to a new owner by applying to Chameleon in writing (as more particularly set out herein) within 60 days following the sale. Failure to transfer the Warranty to a new owner in strict compliance herewith shall result in the balance of the Warranty period being forfeited.

The right to transfer the Warranty is limited to individual homeowners only and under no circumstances shall any transfer be deemed to extend or renew the warranty period, which in all cases shall be a maximum of 50 years (15 years non pro-rated) In the case of corporations, governmental agencies, trusts, religious organizations, schools, condominiums, or cooperative housing, the Warranty is non-transferable, except by a builder to the home purchaser in the event of a new home build.

Should any of the product be determined by Chameleon to have any manufacturing defects or suffer technical failure, Chameleon, in its sole discretion, shall determine whether to replace, restore, or repair and return said product to the homeowner via the authorized ChamClad® retailer/distributor.

To start a claim, a Warranty Claim Form can be filled out and sent to Chameleon. If there are any questions, please contact Chameleon’s customer service at 1 (780) 454 4430 or at info@chamclad.com.

The product is not warranted for suitability of use in all situations. It is the sole responsibility of the purchaser to determine the suitability, effectiveness, and safety of any particular use or application of the product. As building code regulations and conditions vary from area to area, each purchaser should consult local building and safety codes for specific requirements, and qualified exterior contractors to determine the suitability.

## EXCLUSIONS AND LIMITATIONS UNDER THIS WARRANTY

Chameleon’s obligations and liability under the Warranty are expressly conditional upon and subject to the following:

- The installation, storage, and proper handling of the product must have been completed in accordance with ChamClad® installation instructions (please refer to www.chamclad.com for storage, installation and care instructions). Nothing in this Warranty will construe a warranty of workmanship of any installer nor impose any liability of unsatisfactory performance caused by faulty installation, including, but not limited to failing to ensure proper spacing for expansion and contraction. When storing the product prior to installation, it is important that it is not left uninstalled in direct sunlight. Leaving uninstalled product in direct sunlight will void the Warranty.

- The Warranty must have been registered as required herein, and any transferee must have completed and submitted the proper transfer documentation to Chameleon. The claimant must provide Chameleon with all information which it reasonably requires to evaluate the Warranty claim.

- This Warranty does not cover destruction of or damage to the product, or to the product’s failure to adhere to any surface, where caused or contributed to by improper installation, improper cleaning (including the use of unapproved cleaning products or any abrasive materials), misuse, neglect, impact from foreign objects, ice, fire, wind (including tornados or other extreme wind events), earthquake, flooding (overland or otherwise), lightning or acts of God.

- This Warranty does not cover abnormal surface weathering or distortion which may be caused or contributed to by air pollution, fumes, vandalism, misuse, harmful chemicals, or close proximity to heat sources such as fireplaces, barbecues, heaters and open flames. This Warranty does not cover damage to the product’s finish resulting from foreign substances such as grease, oil, sand, and dirt.

- This Warranty does not cover damage to the product caused or contributed to by the reflection, refraction, or intensification of sunlight from nearby highly-reflective surfaces. Care must be taken when determining whether the product is suitable for its proposed application by determining whether there are any reflections, refraction or intensification of sunlight which may arise from windows, doors, or other highly reflective surfaces, including those on adjacent properties.

- This Warranty does not cover discolouration (whether unreasonable or otherwise) or other damage caused or contributed to by air pollution, harmful chemicals and harsh weather conditions. Normal weathering is to be expected as a result of exposure to ultraviolet radiation, atmosphere and weather that may cause staining, fading or other issues which may vary due to the geographic location of a property, sunlight, air quality, or other factors.

- Coverage under this Warranty for discolouration will be limited to abnormal or unreasonable discolouration, judged against the weathering performance provided by Chameleon’s laminate supplier. The standard for unreasonable discolouration in ordinary conditions shall be a colour difference ≥ grey scale 2/3 (ISO 105-A02) for exposure in the US and Canada, and otherwise shall be in the sole discretion of Chameleon. Chameleon may require independent lab testing to evaluate a claim for unreasonable discoloration.

- In the event the product has been painted or sealed using a varnish or otherwise coated over the original factory finish, this Warranty will be void.

- The Warranty will be void if the product is not cleaned in accordance with the approved cleaning methods, as more particularly set out below and at www.chamclad.com.

- ChamClad® products are not warranted against movement distortion caused by structural movement or ground settlement. Use of ChamClad® products beyond normal commercial and residential use in an application not recommended by Chameleon or the local building codes will not be covered.

- Chameleon’s sole and exclusive liability under this Warranty is limited to the repair, restoration or replacement of the product. Chameleon shall not be liable for any other losses, damages, or expenses, whether direct or indirect, incidental, consequential, punitive, exemplary, or otherwise whatsoever, including without limitation, costs of removal or reinstallation of the product, labour, freight, taxes, loss of goodwill, lost profits and loss of use. Chameleon’s total liability whether arising from or based on contract, warranty, negligence, tort, strict liability, or any other cause or basis whatsoever, is strictly limited to the purchase price of the products involved. Chameleon will not be responsible for any loss of any kind arising from a breach of this Warranty, for the sale of the product or from negligence or omissions on the part of Chameleon, its agents or employees. The warranty on any repaired or replacement products will be limited to the unexpired term of the warranty on the original product.

- If a claim is made for a product that is no longer available, Chameleon reserves the right to supply a siding product that, in its sole discretion, is of equal quality and colour range for the purpose of fulfilling Chameleon’s obligations under this Warranty.

- Any person to whom product is supplied under the terms of the Warranty shall only be entitled to benefits of the unexpired term of the Warranty applicable to the product originally installed.

- The Warranty coverage against technical failure is pro-rated as per the below pro-ration schedule (starts from the original date of purchase of the product):

- First 15 years   (0 - 15 years) -100% of the cost of materials will be covered

- Next 5 years   (16 - 20 years) - 75% of the cost of materials will be covered

- Next 10 years (21 - 30 years) - 50% of the cost of materials will be covered

- Next 10 years (31 - 40 years) - 25% of the cost of materials will be covered

- Next 10 years (41 - 50 years) - 10% of the cost of materials will be covered

- If any of the exclusions, disclaimers or limitations are prohibited by law, such provisions may be severed from the Warranty without otherwise affecting the validity of the balance of the provisions.

- This warranty is exclusive and in lieu of any and all other warranties, express, implied, and statutory.

## CLAIM PROCEDURES UNDER CHAMCLAD® WARRANTY

To make a claim under the Warranty, the original owner or registered transferee must notify Chameleon within thirty (30) days of discovering the defect. Such notice must be emailed to info@chamclad.com .The claim must include a description of the defect, date of discovery, the property address where the product was installed, and proof of purchase.

With reasonable notice, the owner shall allow our agents and/or distributor to enter the property and building to view the product only when deemed necessary and solely by ChamClad®. When asked, the owner or transferee will be responsible to provide all documentation and defective samples at the owner’s expense.

CHAMCLAD® WARRANTY REGISTRATION FORM

Please fill out this form completely and include a copy of your invoice (proof of purchase) and email to: info@chamclad.com

- CUSTOMER NAME:

- ADDRESS WHERE PRODUCT IS INSTALLED:

- DATE PRODUCT WAS INSTALLED:

- COPY OF INVOICE FROM DISTRIBUTOR (PROOF OF PURCHASE):

- INSTALLER INFORMATION (NAME AND CONTACT):

## CARE AND MAINTENANCE OF CHAMCLAD® PANELS

ChamClad® products are designed to be low maintenance and our Laminate Films have exceptional properties that ensure ease of cleaning and surface longevity.  They are highly ultra-violet, pollution and salt air resistant.  The film’s low surface tension makes it inherently dirt-repellent and therefore easy to clean.  ChamClad® panel finish includes anti-graffiti properties and in most cases is easily removed with a mild kitchen detergent and warm water. Should you experience removal difficulties, please contact us at info@chamclad.com

ChamClad® panels are cleaned naturally with rainwater. If you choose to clean your panels, use neutral detergent and water.

- Using a soft bristled nylon brush attached to a garden hose, clean the surface with a mild PH neutral soap or non-abrasive kitchen detergent in warm water. DO NOT USE abrasive, erosive, or caustic solvents to clean ChamClad® panels as they can damage the panel finish and could void the warranty.  To avoid streaking start at the top of the wall – working your way to the bottom.

- Follow with a thorough rinsing of fresh water to ensure that substances such as airborne salts, dirt and pollutants are removed.

- Always remove unintentional paint splashes, stucco spills, etc. immediately to prevent the possibility of permanent staining.

- You may pressure wash panels with a low PSI washer. DO NOT exceed 1500 PSI.

Provided that Regular Care and Maintenance practices are followed, ChamClad® Exterior wall panels will provide long term performance and protect your investment.

Customer Service & Support:

Email: info@chamclad.com

Web: chamclad.com

Phone: 780-454-4430

Product Specification Information:

07 46 33 ChamClad® Laminated Film PVC Siding Panels

07 42 93 ChamClad® Laminated Film PVC Soffit Panels

09 54 55 ChamClad® Laminated Film PVC Interior Ceiling Panels

09 78 25 ChamClad® Laminated Film PVC Interior Wall Panel